SCASM - Scenery Assembler 2.36

Disclaimer

----------

SCASM.EXE and all related files come as they are. Use them at your own risk. The author of the SCASM package is not liable for any problems caused by these files.

Copyright (C) 1995-2000 by Manfred Moldenhauer.

e-mail:  100117.1465@compuserve.com

M_Moldenhauer@compuserve.com

Command Reference for SCASM

---------------------------

Every scenery you are creating with this programm has to start with the Header command followed by the LatRange command. These two commands are initialising the minimum file header for further actions. These two commands can only be omitted if you want to create a RAW output file.

After this you can enter any command in any sequence with one exception.

Visual scenery commands have to appear only within area blocks, and in area blocks no other commands are allowed.

Instructions:

-------------

Header( type  [*]  TopLat  BottomLat  RightLon  LeftLon )

-

This command MUST be the first in your (main) scenery

input file. It generates the BGL-file header which

defines the boundaries of the area covered by your

scenery file.

The Header is not needed for RAW BGL output

files -> Area( R ).

-

Lat/Lon values in the format DD:MM:SS.SSSS

type    1...4, this number has something to do with

your *.VIS files.

The following numbers are associated with:

1       visual scenery (sect 9),

synth blocks (sect 1..6)

FS98 AFD-BGL's

2       crop duster world, do not use it

3       navaids as VOR, ILS, NDB, Markers

4       airport menu (FS5 style), ATIS

It seems these numbers shows FS5 how important the

information in this file is and how often to scan

this file for scenery updating. So you should put all

visible things into type 1 and less important things

such as menu and land-me data into type 4 files.

But if you are only designing a small scenery you can

put all into one file.

*       If the optional '*' character is entered the

correct order of RightLon and LeftLon is not

tested. You will need this option only if

your scenery is crossing the date line

(180th meridian). Normaly not used!

LatRange( BottomLat  TopLat )

-

This command has to follow the Header command.

The FS5 scenery is organised in latitude bands and

this instruction defines one of them. There can be

several latitude bands in one scenery file

(.BGL file) but SCASM can only handle one of it per

source file. If your scenery covers a large north to

south distant, lets say more than 2 to 3 degrees

you have to use different source files for each

latitude band.

You then have the option to combine the resulting

.BGL files with the help of SCLINK.

The LatRange instruction is used for all BGL-sections

except for

ground tiles (Seed's/sections 1 to 6),

BGL libraries,

RAW output, and

new facility BGL's (section 20).

Menu( Scenery Title String )                      FS5 - FS95

-

If you want to define some start positions which can

be chosen from the Airport menu use this command.

Length is limited to 20 characters now. Use the

'{' brackets instead of '(' if part of Title String.

Only one Menue() command allowed in this version.

FS98 uses different data structures for the "goto

airport" menu. See the file "FACDAT.DOC" for

generating these new facility data.

To see this 'old fashioned' menus click the

"FS6 and before" button in the FS setup. Also FS98

does not set the navaid frequencies.

MenuEntry( Lat Lon  alt  hdg  COMfrq

NAV1frq  OBI1  NAV2frq  OBI2

Text Info String, max 40 characters )

-

Data for a startsituation. There MUST be a Menu( .. )

command in the file BEFORE you enter a MenuEntry(..)

command, otherwise you got an sequence error from

SCASM. The info string is limited to 40 characters.

Note: Unlike previous versions FS98/CFS do not

support the settings of the radios from this menu.

Also, if you want to use this menu in FS98, you have

to select "FS6 and before" in the "Goto Airport"

menu.

-

Lat     Latitude

Lon     Longitude

alt     Altitude in meter. If set to 0 FS5 will

automatically adjust to ground level.

Floating point numbers accepted.

hdg     Heading (magnetic) in degrees -> 23.33

COMfrq  Frequecny of the COM radio -> 123.40

(first digit ignored)

NAV#frq Frequency of the NAV# radio. -> 116.30

OBI#    OBI# settings in degrees (integer) -> 271

NDB(    Freq   Rng   ID    Mame   Lat   Lon   Elev )

-

This command defines an NDB (Non Directional Beacon).

-

Freq    frequency in kc, range 200.0 ... 526.5

Rng     Range in nm (integer)

ID      5 char NDB id string

Name    24 char Name string, if you want to include

space characters use '_' instead.

Lat     latitude of the position

Lon     longitude of the position

Elev    elevation (MSL) in meters (integer)

VOR(    Frq     Rng     Flags   Magvar  ID      Name

Lat     Lon     Elev    Hdg

)

-

Defines an VOR. For details see -> ILS.

ILS(    Frq     Rng     Flags   Magvar

ID      Name

Lat     Lon     Elev    Hdg              ; localizer

Lat     Lon     Elev    Slope            ; GP

)

-

Defines an ILS (instrument landing system)

-

Frq     frequency in mc -> 113.45

freq. range VOR 108.0 ... 117.975 mc

freq. range ILS 108.0 ... 111.975 mc

Rng     range in nm (integer)

Note: you can enter values up to 255.

It seems FS5 can handle this, but the DME

display will always show "99" if this

value is exceeded.

Flags   any 2 digit hex value is accepted

80 localizer on ON bit

40 glideslope transmitter ON bit

20 localiser back course not available

10 DME at glideslope transmitter position bit

08 DME OFF bit (-> 09 is DME without VOR)

04 voice available (ID ? ATIS ?)

02 TACAN

01 DME availabe bit

example values:

00 or 02 -> VOR

01 or 03 -> VOR-DME, VORTAC

Magvar  magnetic variation (eg.-3.7). Only for VOR.

Normally 0 for ILS. If 0 the VOR/ILS is

aligned with true north.

To get a nav receiver readout which

corresponds to your compass you should enter

the magnetic variation. This gives the VOR

the same error than your compass.

ID      ID string, up to 5 characters

Name    Name string, up to 24 characters.

Use '_' in place of ' '.

Lat     latitude

Lon     longitude

Elev    elevation in meters (MSL, integer)

Hdg     heading (true) in degrees (ILS only,

always 0 for VOR) -> 144.1

Slope   slope angle of GP transmitter -> 3.0

Note: There is a known problem with the slope

in the southern hemisphere in FS5.0 and

FS5.0a.

Remark: Not all ILS systems have an DME equipment.

In the USA DME's usually show the localiser

distance.

In Europe DME's usually show the touch down

zone (TDZ) distance (position of the GP

antenna).

IM(     Lat     Long    Elev    )

MM(     Lat     Long    Elev    )

OM(     Lat     Long    Elev    )

-

Marker bacons:

IM      inner marker

MM      middle marker

OM      outer marker

Lat     Latitude

Lon     Longitude

Elev    elevation in meters (MSL, integer)

ATIS( Rng   Lat   Lon   Frq   RW1 RW2 RW3 RW4 Text    )

-                                       FS5 .. FS98

Defines an Automatic Terminal Information System.

-

Rng     range in km (int)

LAT/Lon ATIS position

Frq     frequency in mc like 123.45

freq. range COM 118.0 ... 137.0 mc

RW1     runway to use for wind directions   0...89°

RW2     runway to use for wind directions  90...179°

RW3     runway to use for wind directions 180...269°

RW4     runway to use for wind directions 270...359°

Text    Any ATIS text you want. The following special

text characters triggers FS5 to send the

following strings:

%A   " weather -",

%B   " observation ",

%C   " ##:## zulu ",

%D   ---

%E   " Temperature ## F -",

%F   " Information ",

%G   " Landing and departing runway # -",

%H   " Advise controler ",

%I   " Altimeter ##.## -",

%J   " Visibility ## -",

%K   " winds $$$$ -",

%L   " Measured ceiling ##### $$$$,",

%M   " on initial contact ",

Note: If you have very long messages you can split

them up into several lines. All <CR/LF> sequences

within the opening and closing brackets of this

instruction are converted into a single space

character. The input line buffer size is about 4000

characters but it seems the current version of FS5.0

can only handle 255 characters. If you need the

'(' & ')' replace them by

'{' & '}'. Also the underscores '_' and the NewLine

charactersare converted to spaces.

NOTE: This command is ignored by FS2K !

LandMe( Lat1    Long1   Elev1   ID1

Lat2    Long2   Elev2   ID2

Airport name Rwy

)

-

This is the data record needed for the 'Land Me'

option in FS5. This option is only available with the

default Cessna.

-

Lat#/Lon#       Defines the positions of the

touchdown points for each end of

a runway.

Elev            elevation of the touchdown points in

meters (MSL). -> 17.3

ID#             runway ID of this touchdown point

TimeZone( TopLat BottomLat RightLon LeftLon minutes ds1 ds2 )

-

Defines a rectangular time zone. Make sure the date

line (180th meridian) is not crossed by the defined

area.

-

minutes         time difference in minutes to UTC

(GMT)

ds1             related to daylight saving time

(start ?)

ds2             related to daylight saving time

(end ?)

-

ds1, ds2 not fully tested, set to 0 if no dst, found

values in original files: 0,0  1,1  2,0  2,1  2,2

1,4  1,3  1,0

GenExcl( name.BGL   mask

TopLat1 BottomLat1  RightLon1 LeftLon1

TopLat2 BottomLat2  RightLon2 LeftLon2

)

-                                       FS5 .. FS98

name    name of the generated *.BGL file

(ie. MY_INCL.BGL)

LL1     Lat/Lon boundaries for the BGL file header;

same as -> Header()

LL2     Lat/Lon boundaries for the scenery (usually

equal to LL1)

mask    16 bit hex value to indicate the CD data to

be suppressed. Any combination of the below

bits allowed.

01  no visual scenery, runways

02  no VOR, ILS

04  no NDB ?

08  no ATIS

other bits not found yet

to exclude all foreign scenery simply enter

F for the mask. It seems not all possible

combinations are allowed.

-

Generates a small BGL file for suppressing specified

FS5.1 CD default areas. It seems theese files have

effects to all other files not in the same scenery

directory than this file.

You can define more than one exclude file for the

same scenerie but be very carefull when defining the

boundaries. Move these file(s) to the same

subdirectory as your other BGL files.

Note 1: you cannot disable any synthetic scenery

tiles (including mountains) with these file(s).

Note 2: The original version of FS6/FS95 has some

problems with these files (The ABOUT MENU for this

version shows Version 6.00 Build 117). Microsoft (TM)

released a patch to solve this problem. The file name

is FEPATCH.EXE.

See -> SC_HINTS.TXT for more information.

Note3: Exclude files do not longer work in FS2K

Visual scenery commands:

The following commands are for the visual scenery section (section 9). Before you can use one of these commands, you have to define an Area block.

No other commands are allowed in such blocks except pseudo commands as Mif, Uvar, Macro ...

Area(   type    Lat     Lon     Rng     )

-

This marks the begin of an block of visual scenery

commands.

type    5, 8, B

This is simply the first byte of the

generated hex code.

The visibility ranges are different for each

type. I do not know the absolute limits.

I have different values from different

sources for them.

5   0  ...  22 km (0 ... 40 km)

8                 40 ... 130 km

(invisible 0 ... 40 km)

B   0  ... 255 km (0 ... 130 km)

I obseved this range can be influenced by the

scaling factor and the V# parameters of the

RefPoint() command.

Lat/Lon location of this area

Rng     range in km units (integer ). It seems this

value controls how long this area needs to

be hold in the scenery buffer. This does not

automatically mean that the objects in this

Area() are displayed, since the visibility

is also controled by the V# parameters in

the reference points (within this Rng limit).

Since it is told FS5.1 can only hold about

256 k bytes of scenery, you should not use

unrealistic large values to avoid wasting

memory.

Note! In FS5 a single scenery area is limited to

16kB. SCASM 1.6g/1.67 (and above) is testing this

limit and will produce an error message, but the

scenery is compiled correctly up to about 24..32 Kb

(depending on the current buffer status).

FS98 is able to handle larger objects. Use SCASM's

SET() command to enable larger object compilation.

EndA

End_of_Area

This marks the end of an Area block. No code is

generated. You can see this command as a special

jump destination. All unresolved Jump()s and all

'empty' labels (-> only ':') are directed to this

location.

This command also causes some internal actions such

as patching all label references and writing buffers

to disk.

Area( R )

-

This is an option to produce a raw BGL output of

visual scenery objects.

To enable the RAW output you also bave to call SCASM

with the -raw command line switch or the Set( raw 1 )

pseudo command to suppress the normal BGL file

header.

Also no Header() and LatRange() commands are needed.

The RAW output file cannot be used directly in FS and

also SCLINK cannot handle it.

The output file starts with a long integer number

(4 bytes) which indicates how many valid bytes of BGL

data will follow.

SCASM can only compile one "raw" object per compile

run. A typical source file for raw output will look

like:

Set( raw 1 )

Set( areamx 64 )

Area( R )

...

... ; your BGL instructions

...

EndA

RefPoint( type :Label scale Lat Lon [...] )

-

This command defines a reference point for further

actions. All distances entered now are relative to

this point. If FS5 decides the viewer is too far

away, a jump to :Label is performed and the following

commands are not executed. For the Refpoint type

selector I simply chose the first digit of the

generated hex code.

The optional v1= and v2= parameters may help the

drawing systen to speed up things in dense sceneries.

They are used to control the visibility of objects.

-

type    type of the RefPoint. There are different

types of RefPoints as follows:

2       or

abs     absolute referencepoint, MSL (main

sea level) This type is used if an

absolute elevation setting is needed.

The elevation is set with the "E="

parameter. The scenery object is

put on this level, regardless what

the altitude definition in the

coresponding synth tile says.

7       or

rel     relative referencepoint, always 0 AGL

The altitude is adjusted to the same

level as defined in the Synth Tile

block you are standing on. The "E="

parameter is ignored.

3       or

ns      Reference point with NO SCALE

setting. This type is often used for

very detailed and complex objects.

The altitude setting for this

referencepoint type is always

absolute (=MSL).

For setting the scale factor use the

-> SuperScale() / SetScaleX()

command.

:Label  symbolic name of a jump destination

scale   scale factor, if scale = 1 all distances are

in meters. Floating point values accepted.

This parameter is not allowed for type "ns".

Lat/Lon Position

these optional parameters are prefixed by an

identifier

E= ###  elevation of this point in meters (MSL),

not for type 7/rel. Always required for type

"abs" and "ns".

v1= ##  visibility range of this object in meters

(int)

v2= ##  radius of this object in design units (int).

This is in meters if scale factor is 1.

-

Some other special reference point formats are:

RefPoint( nsi :Label varptr [...] )

no scale with indirect position via varptr.

(v1= ## v2= ## optional) For scaling use

SuperScale()

RefPoint( ind :Label scale varptr [...]

reference point with indirect position via

varptr. (v1= ## v2= ## optional)

RefPoint( ind :Label scale var [...] )

-

This reference point is used in the aircraft model

files and uses an indirect position definition.

-

ind     the keyword "ind" indicates that indirect

positioning is used.

var     the address of the variable area that holds

the current position (Lat, Lon, Alt).

ShadowPos( Lat Lon ALt )

-

This command sets the position for a shadow of an

static object which is not on the ground like the

blimp in thc Chicago scenery.

ShadowPos(...)

SuperScale(...)

ShadowCall(...)

ShadowPosInd( var )

-

(SHADOWVPOSITION in FS98SDK)

This command sets the shadow position of an non

static object like an aircraft. Since the position is

variable in this case it is addressed indirect by

a pointer to an array of variables.

Used in .MDL files and dynamic object libraries.

ReScale( :Label V1  V2  scale )                 -> FS98 SDK

SetScale( :Label V1  V2  scale )

-

This command sets/changes the scale factor for normal

reference points (abs, rel).

If the visual range test parameters are used, a range

test is done.

-

V1, V2  visual range test parameters (used if not 0).

Same as in the reference points.

:Label  jump to this label if range test is negative.

scale   scale factor (decimal, floating point value)

SuperScale( :Label v1  v2  sx )                -> FS98 SDK

SetScaleX( :Label  V1  V2  SX )

-

Sets a sort of binary scale factor for reference

points types ns and ShadowPos() .

-

V1, V2  visual range test parameters (used if not 0).

Same as in the reference points.

:Label  jump to this label if range test is negative.

SX      This is actualy the exponent of the scale

factor (integer).

True scale factor is calculated as follows:

scale = ( 2^SX ) / 65536

example:

scale = ( 2^16 ) / 65536 = 1

As a result the scale factor can only be set

to a value which can be expressed as a power

of 2 (devided by 65536).

Another way to calculate scale is:

scale = 1 / 2^(16-sx)

with sx = 7 this results in:

scale = 1 / 2^(16-7)

= 1 / 2^9

= 1 / 512

Maybe this sort of scaling speeds up the

internal calculations because the scaling

can now be done by shifting. Do not confuse

with 2*10^sx .

Points( firstnum        x1 z1 y1  ...  xn zn yn )

-

This command builds a table of 3D points. Normaly

firstnum should be set to 0. Other values are used

to change or expand an existing list. The number of

points per area block is limited to 400 (200 in older

versions, 800 in a 32-bit vers.).

DefPoint( index  x  z  y )

-

Defines a single point in the list of points. Note,

points defined with this istruction cannot be used

with SCASM's automatic vector calculation feature

unless this command is only used for updates.

-

index   the index number of this point

VecPoints( m     firstnum x1 z1 y1  vx1 vz1 vy1 ...

xn zn yn  vxn vzn vyn )

VecPoints( p     firstnum x1 z1 y1  el1 hd1     ...

xn zn yn  eln hdn )

VecPoints( vattr firstnum x1 z1 y1 ... xn zn yn )

-

This command builds a table of vector points. The

vectors are used for color shading with

-> ShadedPoly(). If you have unregular shaped objects

it may be better to calculate the vectors manually.

In automatic mode the number of points are limited

to 400 in this version. The automatic function gives

the vector the direction from the RefPoint TO this

point, but remember this is correct only for regular

shaped objects. It is possible to suppress ONE vector

component by using xy xz yz vattr. Other values may

confuse the assembler. For example you may use

"xy"-flags for a cylindric gas tank.

-

x#  z#  y#      coordinates of a point (integer)

vx# vz# vy#     components of a vector

firstnum        number of first point (usually 0).

vattr           vector calculation attributes, see

above. (a, xy, xz, yz, p)

el#             elevation angle in degrees

(-90° .. +90°)

hd#             heading angle

AutoPoints( firstnum numcount   dx1 dz1 dy1   dx2 dz2 dy2 )

-

This command draws nothing. It only calculates and

fills a table of points. Numcount points are

generated, where firstnum is the first one. You can

think of this being a command to calculate the

coordinates for a dotted line from P1 to P2 and than

fills the point list.

Note: In original sceneries this command was not

found with a Numcount value above 7.

Points defined with this command cannot be used with

SCASM's automatic vector calculation feature!

MoveToPt( # )

-

Moves the drawing cursor to the 3D-coordinates given

by the point #(number) of a list of 3D points which

is previously defined by the Point() command.

For lines and surfaces.

-> Points(), ->VecPoints(), -> StartSurface,

-> EndSurface, -> DrawToPt(), ConcavePoly

DrawToPt( # [#...#] )

-

A drawing command from the previous point to this

point# is executed. The points has to be defined in

an 3D point list.

This command is repeated for every # in the brackets.

StartSurface

-                                        FS5 - FS98

not for FS2K

If this command is entered, you can use the above

commands to define a surface (covered with color or

bitmap).

-> EndSurface, -> SurfaceColor(), -> ConcavePoly,

-> Bitmap()

EndSurface

-                                        FS5 - FS98

not for FS2K

This indicates the end of an surface drawn with the

MoveToPT() and DrawToPt() commands.

MoveTo( x z y )

-

This command moves the drawing cursor to the 3D point

which is defined by the given coordinates.

For lines only. Do not mix the MoveTo()/DrawTo() type

of instructions with the MoveToPt()/DrawToPt()

instructions and also do not use it for 'shadowed'

objects.

-> LineColor(), -> Brightness(), -> DrawTo()

DrawTo( x z y )

-

A drawing command from the previous point to this

point is executed. For lines only.

-> LineColor(), -> Brightness(), -> MoveTo()

LineColor( num attr )

-

Sets the color for lines and dotted lines and single

dots.

ShadedColor( num attr )

-

The color for shaded polygons is set. It seems this

command also sets the surface color.

SurfaceColor( num attr )

-

num     color code (hex)

attr    color attributes

F0      normal colors, changes with time of

day

68      transparent colors

69      direct palette color

-

Sets the color for normal polygons and surfaces

defined with

-> StartSurface - Move..() - Draw..() - Endsurface or

-> Poly()

LoadSurfaceColor( FSvar )

-

The surface color code is loaded from a local

variable.

LoadShadedColor( FSvar )

-

The shaded color code is loaded from a local

variable.

LoadLineColor( FSvar )

-

The line color code is loaded from a local variable.

Bitmap( name.ext  x  dx  dz  dy )

-                                        FS5 - FS2K

Load a bitmap (texture) file from the active TEXTURE

directory.

in FS5.0 and FS5.0a this is usually:

C:\FLTSIM5\TEXTURE

in FS5.1 this is could be:

C:\FLTSIM5\TEXTURE            general/global

texture

directory

_OR_

C:\FLTSIM5\MY_SCEN\TEXTURE    the local

active scenery

texture

directory

-

Bitmap files often have extensions '.R8' but there

are also bitmaps with other extensions. I think the

'.R8' extension should be reserved for the "global"

textures which are stored in the genaral texture

directory. Therefore I recommend to use your own

extension if your texture is not supposed to be

shared with other sceneries. However, some authors

and users prefer to use always the .R8 extension for

easier recognition of texture files. Therefore it is

important to mention these extensions in your scenery

documents.

-

name.ext        The name of the bitmap file. You

cannot specify any drive or path name

in this instruction

(limit in BGL file format).

dx              x offset in RefPoint units

dy              y offset

dz              z offset, normaly 0

x               unknown (decimal).

Often found values: 0, 1, 8

Note:

The bitmaps for FS5 are a simple 256 x 256 pixel

(/texel) arrays using 1 byte per pixel. Note the

following x, y pixel adress:

top left          0   0

top right       255   0

bottom left       0 255

bottom right    255 255

For some reasons bitmaps drawn with

Bitmap() - StartSurface - MovetoPt() - ... -

Endsurface

are drawn top down !

The standard color palette is stored in FS5.PAL (in

FLTSIM5\TEXTURE dir). You cannot view/edit these

bitmaps with normal drawing programms but there are

several "R8" editors available in flight simulation

related BBS's.

RepeatBitmap( dx  dz  dy )

-                                        FS5 - FS2K

Repeat/reactivate a still loaded bitmap. Parameters

are the same as in Bitmap().

BitmapMode( first_color )

-

preliminary, name may be changed in next versions

(Bitmap copy mode)

It seems this command controls the handling of the

pixels in a bitmap. This command sets the first color

index number which is copied ito your scenery.

Use this instruction only with TexPoly().

-

first_color     Decimal value 0 .. 255. This is the

first color number to be copied.

Do not forget to change back to 0

(normal). All colors from 0 to this

value appearing to be transparent.

Note: It is reported to me that this command causes

problems if used with the new LoadBitmap() command.

So use it only with the old FS5 Bitmap() command.

Inst_81( state )        ; old, not longer supported

AntiAlias( state )      -> FS98 SDK

Smoothing( state )

-                                        FS5 - FS2K

This instruction is found very often just following

the Bitmap() instruction. It seems this instruction

controls the image smoothing for ground textures. It

comes into effect when the option image smoothing is

set in the preferences display option menu.

-> TexWindow(), -> ShadedTexPoly()

-

state   0       off

1       on

MipMap( state )                                     -> FS98

-

state   0       off

1       on

-

This instruction switches mipmapping on/off.

This command is mentioned in the FS98 SDK but

doesn't seem to work as expected.

Transparency( mode )                            2.11 / FS98

AlphaColor( mode )                               -> FS98SDK

-

Different modes of transparent color effects are set.

Use this command with the normal Poly() command if

you want a transparent polygon. This is the propeller

disk effect in FS98. (alpha blending)

Note, it is very important to switch this mode off

after drawing the last Polygon.

Otherwise a page fault in the FS98 module HG2D.DLL

may happen.

-

mode    hex number:

0   transparency mode OFF. Don't forget to

use this command to switch back to normal

color mode.

1   nearly complete transparent

...

F   highest density in transparent mode, not

longer transparent since the result is

always gray.

1x  different modes of "funny" color effects

-

Example:

Area( 5 Lat  Lon  5 )

PerspectiveCall( :alpha )

Jump( : )

;

:alpha

Perspective

RefPoint

( rel :endobj 1 Lat Lon v1= 5000 v2= 100 )

Points( 0

-20 0  0

-20 40 0

20 40 0

20 0  0

)

AlphaColor( 6 )

Poly ( a  0 1 2 3 )

Poly ( ai 0 1 2 3 )

AlphaColor( 0 )    ; important !!!

:endobj

Return

;

EndA

MoveTexture( x1 y1 x2 y2 )

TextureBounds( x1 y1 x2 y2 )    -> FS98SDK

-

TextureEnable( s )                                     2.20

-

s      state s = 0 disabled

s = 1 enabled

-

This instruction disables or enables an texture.

Mainly this instruction is used to disable a texture

after drawing a textured river or an textured

taxiway.

Palette( name.ext )

-

The color palette file 'name.ext' is loaded. You

cannot specify a drive or path name since this BGL

code only supports 8.3 DOS name convention.

You do not need this instruction if you are using the

FS5 default color palette for your bitmap/texture

files.

Note, if you are using FS5.1 you will also need a

corresponding *.HAZ file otherwise you cannot use the

haze option with your scenery.

CustomColors( dec_num )

-

This command is used to reduce the number of

customised colors (normally 64) in the active color

palette to gain some space for special night color

effects. You must have ( or generate)

a matching color palette file.

Brightness( 1 - 100 )

-

This command sets the brightness for a color. The

range is from 1.0 to 100.0 percent.

-> LineColor(), -> Surfacecolor()

& Startsurface .. Endsurface

DotLine( x1 z1 y1   x2 z2 y2   num )

-

A dotted line from point(x1 z1 y1) to

point( x2 z2 y2 ) is drawn. The number of dotts is

num. The color of the dot is selected with

-> LineColor()

Dot( x z y )

-

A single dot is drawn at the point(x z y ). The color

of the dot is selected with -> LineColor()

DotPt( # )

-                                    not for FS2K ?

A single dot is drawn at the point with the

number # of a predefined list of points. The command

is repeated as many times as there are dot numbers in

the brackets.

-> LineColor(), -> Points()

Ball( size x z y )                              -> FS98 SDK

BigDot( size x z y )

-

A big dot is drawn at the position defined by the

x y and z coordinates. This command is used for some

interesting light effects. Set the color with

LineColor() AND SurfceColor().

The function of the size parameter is not absolutely

clear now. Recommended value for size is 512. Format

of the size parameter may be changed in future

versions.

ConcavePoly

Concave              -> FS98SDK

-

Indicate to the drawing system that a concave polygon

follows. Does not work with bitmaps in FS5.0.

-> Poly(), -> StartSurface - Move..() - Draw..()

EndSurface

Poly( vattr  [vx vz vy   len]   pnum1 ... pnumn )

Poly( m       vx vz vy   len    pnum1 ... pnumn )

Poly( a                         pnum1 ... pnumn )

-

A polygon is drawn with the points pnum1 to pnumn of

the actual point list and with the actual surface

color.

Note: All polygons are only visible only from one

side. The vector parameters indicate the direction

from where the polygon can be seen. The brightness

of the used color varies with the angle of the light

source (the sun, day of time).

Note! It seems FS5 cannot handle more than 100

vertices in any single Polygon. Therefore SCASM now

generates an error message on every ..Poly..()

command if more than 100 vertices

are detected. (in FS98 the limit may be 80 vertices)

-> SurfaceColor(), -> Points()

ShadedPoly( vattr [vx vz vy  len]   pnum1 ... pnumn )

ShadedPoly( m      vx vz vy  len    pnum1 ... pnumn )

ShadedPoly( a                       pnum1 ... pnumn )

-

A shaded polygon is drawn. The shading requires 3D

points defined with the VecPoints() statement and

ShadedColor(). The shading is used to get a smoth

change of the brightness from one polygon to the

next. This can make an octagonal object

looking perfectly round. -> ShadedColor(),

-> VecPoints()

TexWindow( vattr [vx vz vy  len]   pnum1 ... pnumn )

TexWindow( m      vx vz vy  len    pnum1 ... pnumn )

TexWindow( a                       pnum1 ... pnumn )

-                                         FS5 only ?

pnum#   number (index) of a point from the point list

-

Defines a window through which a bitmap is shown.

The window should not be concave.

Do not use it for FS6/FS95 or later sceneries !!

TexPoly( vattr [vx vz vy len] pnum1 bx1 by1 ..pnumn bxn byn )

TexPoly( m      vx vz vy len  pnum1 bx1 by1 ..pnumn bxn byn )

TexPoly( a                    pnum1 bx1 by1 ..pnumn bxn byn )

TexPoly( at                   pnum1         ..pnumn         )

-

Draws a polygon covered with a texture bitmap. The

points has to be defined with the Point() command.

The polygon is always drawn with the same brightness,

regardless to the location of the sun. The brightness

can be changed using TexPolyShading() if the bitmap

is prepared for that (see -> TexPolyShading() ).

The bitmap-point bx1-by1 is tied to ptnum1 and so on.

Image smoothing does not work with this command.

Color shading can be done with TexPolyShading() and

a special prepared bitmap.

-> Bitmap(), -> Points(), -> TexPolyShading()

-

pnum#   number (index) of a point from the point list

bx#     bitmap x coordinate (0...255)

by#     bitmap y coordinate (0...255)

t,b,r   -> see vattr, texture automatic

-> see also annex 9 for more information

TexPolyShading( vx  vz  vy )

TexPolyShading( p   el  hd )

-

This instruction sets the shading intensity of

building bitmaps (side#.R8) in conjunction with

textured polygons (->TexPoly() ).

This shading requires a special prepared bitmap file

with 8 areas one for each of the 8 intensity steps).

-

vx      x-vector component

vz      z-vector component

vy      y-vector component

p       format flag -> polar coordinates, vector will

be converted to FS5 format by SCASM.

el      angle of elevation

hd      angle of heading

ShadedTexPoly

( vattr [vx vz vy len] pnum1 bx1 by1 .. pnumn bxn byn )

ShadedTexPoly

( m      vx vz vy len  pnum1 bx1 by1 .. pnumn bxn byn )

ShadedTexPoly

( a                    pnum1 bx1 by1 .. pnumn bxn byn )

-

Draws a shaded polygon covered with a texture bitmap.

The points has to be defined with the VecPoints()

command.

The bitmap-point bx1-by1 is tied to ptnum1 and so on.

The shading is done similar to ShadedPoly().

Bitmap colors above 127 are shown as black!

->VecPoints(), ->Bitmap()

-

parameters see -> TexPoly() command.

VecPoly

( vattr  [dx  dz  dy   vx  vz  vy]   pnum1 ... pnumn  )

VecPoly

( m       dx  dz  dy   vx  vz  vy    pnum1 ... pnumn  )

VecPoly

( a                                  pnum1 ... pnumn  )

-

A polygon is drawn with the point pnum1 ... pnumn of

the actual point list and with the actual surface

color.

This polygon command is very similar to the normal

Poly(...) command. It seems the only difference is

the origin of the visibility vector. In auto mode

SCASM copies the coordinates of the first point

(pnum1 -> dx,dz,dy) as the origin point of the

vector and then calculates the vector components

(vx,vz,vy) as usual.

TexVecPoly( a   pnum1   bx1  by1   ...   pnum#  bx#  by# )

TexVecPoly( vattr   dx dz dy    vx vz vy

pnum1   bx1  by1

...

pnum#   bx#  by#

)

-

This is the textured VectorPolygon (-> VecPoly() )

This instruction draws a polygon using the points

pnum1 to pnum#. Like other textured polygon commands

each point is combined with a specific pixel of a

bitmap.

-

vattr           vector attibutes (i.e. 'a') to

indicate how SCASM should handle

vector data.

a       automatc mode,

dx,dz, dy, vx, vz, vy are

taken/calculated from the

first 3 points.

m       manual mode, all vector data

must be entered.

t,b,r   -> see vattr, texture

automatic.

dx, dz, dy      offset koordinates of a point on the

polygons surface. You can chose one

of the points defining this polygon.

vx, vz, vy      The 3 vector components.

pnum#           The number (index) of a point

(defined with Points(..) )

TexRelief(  x y  wx wy  dx dy

px  py  alt

...

px  py  alt

)

ElevationMap( ... )    -> FS98 SDK

-

A loaded bitmap is used to draw a textured relief on

the ground. For this purpose a regular grid of

rectangles is defined and each grid node is bound to

specific altitude and texture point. Note, you are

specifying the number of sides for the rectangels and

the number of altitude points MUST match.

The smallest grid is defined by X=1 and Y=1 and has

4 grid points (vertices).

This instruction should be used in a

PerspectiveCall-ed subroutine. Be carefull; this

command can be a real frame-rate killer.

Note: Since this instruction also includes

crash detection you should use it only with

RefPoint( abs ... ) and a corect altitude setting.

-> PerspectiveCall, -> PerspectiveCall2()

-

x, y    amount of grid rectangles in x and y

direction

wx      width of an grid rectangle in x direction

(east-west)

wy      width of an grid rectangle in y direction

(north-south)

dx      x position offset of texture pixel 0,

left bottom corner

(related to the refpoint)

dy      y position offset of texture pixel 0

left bottom corner

(related to the refpoint)

Note: Pixel 0 is the south-west (bottom left)

corner of the bitmap.

px      texture pixel x coordinate

py      texture pixel y coordinate

alt     altitude value for the given pixel and the

current grid point (integer)

Jump( :Label )

-                                               ALL

Execution is continued at :Label

The jumptable is deleted everytime a new Area() is

started, so you can use the same name in different

areas. This also means you cannot jump out af an

Area()->EndA block.

Note: If not noted otherwise, all jump's and call's

are using relative adressing with a 16 bit

displacement.

Jump32( :Label )

-                                     FS98/CFS/FS2K

Jump with a 32 bit address displacement

Mainly used in aircraft model files (.MDL)

VectorJump( :Label m  vx vz vy len )

VectorJump( :Label a  pnum1 pnum2 pnum3 ... )

VectorJump( :Label p  elev_angle  heading  len )

SeparationPlane( ... )                           -> FS98 SDK

-

The elements vx, vz and vy are representing a vector

standing rectangular on a virtual plane (surface).

Len is a distance in RefPoint-units. The following

commands are executed if the distance from the viewer

(aircraft) to the virtual plane measured in the

vectors direction is greater than 'dist'.

Otherwise execution is continued at :Label (Jump to

:Label).

In some cases the plane is identical with one of the

surfaces of an object you have just constructed. If

so you can use the auto-vector feature to calculate

the vector and the distance.

You have to enter only 3 point numbers of the object

surface (polygon). If there are more, the others will

be ignored.

-

Note: This instruction has the same function as the

GLUE Template in the FSFS aircraft factory.

IfVarRange( :Label      Var minval maxval )

-

The contents of the FS5 variable Var is tested. If

the value is within the range of minval and maxval

the execution is continued with the next command.

Otherwise a jump to :Label is performed.

-       Var     FS5 variable number in HEX.

minval  minimal allowed value (decimal/or 0x. hex)

maxval  maximum allowed value (decimal/or 0x. hex)

IfVarRange2( :Label     Var1 MinVal1 MaxVal1

Var2 MinVal2 MaxVal2

)

-

conditions ANDed

IfVarRange3( :Label     Var1 MinVal1 MaxVAl1

Var2 MinVal2 MaxVal2

Var3 MinVal3 MaxVal3

)

-

5.1 only !

IfVarAnd( :Label Var Mask )

-

The contents of the FS5 variable Var is AND_ed with

the value of Mask. If the result of this operation

is TRUE execution is continued with the next command.

Otherwise execution is continued at :Label.

-

Var     FS5 variable number in HEX

Mask    Bitmask, any 16 bit HEX value

IfHRes( :Label r p )

IfHSize( :Label r p )   -> FS98 SDK

-

The horizontal size/resolution is tested. If the

object size given by r does not cover p pixels a

jump to :Label is performed and the object is not

drawn. But you can force FS to draw it just by

increasing your zoom factor during flight.

The effect is like using binoculars.

-

r       object radius, integer

p       pixels, integer

IfVRes( :Label r p )

IfVSize( :Label r p )   -> FS98 SDK

-

The vertical size is tested.

-

r       object radius, integer

p       pixels, integer

SetVar( FSvar val )

-

FSvar   FS5 local variable number in HEX

val     new value in decimal (or hex if started

with 0x## )

Please use upper case characters for hex

numbers.

SetVar7E( FSVar val )

-

another SetVar instruction. Only found in some add-on

sceneries. The difference to normal SetVar() is

unknown.

Call( :Label )

Call32( :Label ) ; version for 32 bit offset (FS98+)

-

Calls a subroutine at :Label. After the Return from

the subroutine execution continues with the next

command. -> Return

Return

Returns the control to the next instruction after the

..Call() command. (Return from subroutine).

Every subroutine MUST end with a Return command.

All parts entered with a ..Call.. command should be

considered as a subroutine.

Most FS5 'database error' mesages are caused by

forgoten Returns or misplaced Jump instructions.

-> Call(), -> ..Call()

PerspectiveCall( :Label )

AddObj( :Label )   -> FS98 SDK

-

Use it for drawing all 3 dimensional objects. This

instruction causes  FS5 to determine which objects

or part of objects are hidden and which not. There

is no need to draw flat ground surfaces (color or

bitmap) with this instruction.

Typical sequence:

;

Area( ... )

PerspectiveCall( :House )

Jump( : )

:House

Perspective

RefPoint( :H_end ... )

SurfaceColor(...)

Poly( ... )

...

:H_end

Return

...

EndA

AddMountain( :Label )           -> FS98SDK

PerspectiveCall2( :Label )

-

implemented for testing, name may be changed in

future versions.

Perspective

-

This should be the first command in a subroutine

called with PerspectiveCall(). If you forget this

command you will see nothing or even crash !

ShadowCall( :Label )

-

The shadow of an static object at :Label is drawn.

Do not direct a ShadowCall to a Perspective command,

use the next command instead. Otherwise scenery

hangups can happen.

-> Return, -> see Building(...) example

ShadowCallVI( :Label  var )

-

SHADOWVINSTANCECALL in FS98SDK

This command is used to draw a shadow of variable

objects. The position is specified in an array of

variables. Used for dynamic objects.

RotatedCall( :Label xdeg ydeg zdeg )

-

The subroutine at :Label is called with rotated

coordinates. The pivot point is the active

RefPoint().

-> Return

TransformCall( :Label  delta_x  delta_z  delta_y

x_deg  xvar

y_deg  yvar

z_deg  zvar  )

-

This is an expansion of the RotatedCall().

A subroutine is called with transformed coordinates.

That means you can assume the new temporary reference

point is at the given delta_# distances from the

original point. Also the coordinate system is rotated

around this point according the #_deg values.

Note: Shadows produced from the transformed scenery

element will also be moved. This can cause funny

flying shadows.

-> Return

-

:Label  Address of the subroutine which is called

with a transformed coordinate system.

delta_# linear displacement of the object (integer)

#_deg   rotation in degrees

(floating point, i.e. -22.5)

#_var   variable (hex), 0 if not used !

CallAsm( :Label seg )

-                                   FS5, FS5.1 only

Calls an 8086 assembly language programm at :Label

I have no information about the seg (hex) parameter.

Set it to 1. The assembly language instructions can

only be entered using the Dbx(...) binary

instructions family.

FS5.x only, ignored by FS6.0/FSFW95 and later to

avoid conflicts due to the new 32 bit environment.

-> FS98SDK

RoadStart   ( width   delta_x  delta_z  delta_y )

RiverStart  ( width   delta_x  delta_z  delta_y )

TaxiwayStart( width   delta_x  delta_z  delta_y )

RoadCont    (         delta_x  delta_z  delta_y ... )

RiverCont   (         delta_x  delta_z  delta_y ... )

TaxiwayCont (         delta_x  delta_z  delta_y ... )

-> old

RoadMoveTo(     width   delta_x  delta_z  delta_y )

RoadMoveTo2(    width   delta_x  delta_z  delta_y )

TaxiMoveTo(     width   delta_x  delta_z  delta_y )

RoadLineTo(             delta_x  delta_z  delta_y ... )

RoadLineTo2(            delta_x  delta_z  delta_y ... )

TaxiLineTo(             delta_x  delta_z  delta_y ... )

-

Every ..Move../..Start instruction sets the starting

point of a road or taxiway relative to the active

reference point. The ..Line../..Cont instruction

draws a road or taxiway from the previous point to

the point defined in this instruction and stores his

position as a starting point for an following

..Line../..Cont instruction.

During dusk and nights roads have orange lights and

taxiways have blue lights. The color can be chosen by

-> SurfaceColor().

If a texture is loaded just before these drawing

commands the river or taxiway is drawn with this

texture. You can use TextureEnable( 0 ) to disable

this texture.

-

width   1/2 width in refpoint units (integer)

Negative values can be used to switch off the

road/taxiway lights.

delta_x distance in east-west direction from

reference point or from the previous point

(integer).

delta_z height difference, usually 0 (int)

delta_y distance in north-south direction from

reference point or from the previous point

(integer).

Note!   The ..LineTo() commands are repeated as many

times as there are coordinate triples in the

brackets.

Both name versions are available and generate the

same code. The new names are added for better match

with the FS98 SDK.

Important note: For FS2K use RRStart() for roads and

rivers.

Runway related commands:

------------------------

LayerCall(  :Label layer )

-

Ground surfaces drawn within a LayerCall'ed

subroutine are sorted according to their layer number

before they are drawn into the scenery. The sorting

is done with all currently visible scenery elements,

even if they are in different BGL files. This calls

one or more RunwayData records -> see example.

Please see the example for the RunwayData()

instruction.

-

layer   number (1 to 63 decimal) of a priority layer

(category layer in FS98SDK) for a flat

surface polygon to cover the ground. Polygons

with higher number are always drawn on top of

the others.

Use the following layer / category numbers:

4      Mesh, seed, ground

8      ground polygon / texture overlays

12      river

16      road

20      lines, strobes

24      runways, taxiways

28      mountain

60      shadows

Note: It is reported that only layer numbers

4, 8, 12, 16 ... are working in FS2K

RunwayData( Lat Lon  <List_of_parameters_in_any_sequence> )

-

Runway data record. the Lat Lon parameters must be

the first one (without any keyword). All other

parameters are entered in any sequence and are

identified by the keyword. There must be a space

character between the keyword and the data. The

capital letters N and F are representing the

Near/Far end data. The near end of the runway is that

one nearest to you when landing on that end which

corresponds with HEADing. Default value for all

omitted fields is 0. Runway data are usually embedded

in type 5 Area() blocks.

Note, this instruction changes the scaling factor!

-

ALTitude        runway altitude in meters MSL

(floating point values accepted).

Please note, you are only drawing a

picture of the runway at this

altitude. You can fly through

this picture if this value does not

match with the ground level. Use

Synth blocks to adjust the ground

level !

HEADing         true heading of the runway in degrees

(float)

LENgth          runway length in ft (integer)

WIDth           runway width in ft (integer)

ID              runway ID number, additional

characters [L|R|C] are allowed.

ID numbers are shown only if the

ID-bit in parameter "Markers" is set!

SURFace         number of surface type

0       mud,

1       concrete,

2       asphalt,

3       grass

Markers         Two digit hex value. Add the

corresponding values of the required

feature.

01      white border line (Edge Line) at the

left and right side (no effect on

FS5.0)

02      treshold marker bit, enables the

'piano key' painting at the runway

ends.

04      touchdown markers, enables the touch

down zone marking 150m (492ft) from

the runway threshold.

08      Fixed distance marker, enables 2 fat

distant markerings 300m (984ft) from

the runway threshold.

10      center line

20      ID number

40      distance markers, enables distant

markings about every 150m. This bit

also enables the color change to

yellow for the far away runway edge

lights.

80      ???

Lights          One digit hex value, controls the

ligths at the sides of the runway.

00      off

01      low intensity light

03      high intensity light

04      centerline lightning on

The following parameters are available for both ends

of the runway. These ends are identified by the last

letter

N=near / F=far end.

The near end is defined by the "Heading" parameter.

It is that end of the runway near to you when

aproaching at the given heading.

ThrLightsN ThrLightsF   Hex value

01 enables the green/red

treshold lights

04 enables REIL (Runway End

Identification Lights,

white flashlight, ignored

by some older versions).

You MUST switch on the

threshold light if you want

aproach lights or VASI.

StrobesN   StrobesF     number of strobes (dec)

VasiSlopeN VasiSlopeF   VASI slope in degrees

VasiSideN  VasiSideF    VASI distance from the center

line in ft (int). To indicate

the side you can use the

characters R or L instead of

a -/+ sign

(i.e. VasiSideN L120 ). If

there is no L/R SCASM assumes

the VASI on the left side.

The side is always indicated

from the pilot.

VasiDistN  VasiDistF    VASI distance from the middle

of the rwy in ft. No sign is

needed !! A '-' sign will

reverse the direction. For a

2-bar VASI this distance is

measured to the middle of the

2 bars. This distance should

correspond to the touch down

markers (->Markers) and the

ILS-GP.

VasiSpaceN VasiSpaceF   Spacing between the VASI

elements in ft.

VasiBarsN  VasiBarsF    Number of VASI bars.

0       no VASI

1       2 bars

2       3 bars

AprLN      AprLF        Aproach lights type(decimal).

Be sure you have threshold

lights enabled.

0       none

1       syncronous flashing

strobes

2       MALSF

3       MALSR

6       ALSF-1

7       ALSF-2

8       running strobes

9       similar to 10,

without the red

lights crash

(FS5.0 german

version w/o update)

10      ICAO III

(ignored by

FS5.0 w/o update)

ThrOffN    ThrOffF      Offset distance in ft for

displaced thresholds. This

section is subtracted from

the runway length and is

painted with white arrows.

Ignored by older versions.

This parameter changes the

threshold light pattern !

ExtN       ExtF         Runway extension. This

pre-threshold area is

painted with a yellow chevron

marking pointing to the

threshold. This length is

added to the runway. This

area is not available for

taxing and landing. This

parameter changes the

threshold light pattern !

This is a typical sequence for a runway block:

;

Area( 5  N54:11:13  E07:55:00  15 )

LayerCall( :runways 24 )

;

Jump( : )

; note! the Label is omitted, this is a Jump

;  to the end of block

:runways

;

; first runway record

;

RunwayData( 54:11:17.0493 07:54:57.7855

altitude 3.5

heading  57.2

lenth    846

width    40

surface  1

)

;

;  next runway record

;

RunwayData( 54:11:16.2394 07:55:00

alt        3.5  heading    27.2 id      3

lenth      4788 width      98

surface    2    lights     13   markers 36

THRLightsF 1    THRLightsN 1

VASIBarsF  1    VASISpaceF 330

VASIdistF  1115

VASIsideF  r120 VASIslopeF 3.0

VASIBarsN  1    VASISpaceN 330

VASIdistN  1115

VASIsideN  L120 VASIslopeN 3

ThrOffN    740  ThrOffF    740

)

;

;  last runway record

;

RunwayData( 54:11:10.4085 07:54:58.0623

alt     3.5     head 327.2      len 1312

wid     98      surf 1          id 33

markers 32

)

Return

;

; Do NOT FORGET this Return !!!

; =============================

;

EndA    ; this marks the end of the Area block

RunwayLights( <list_of_parameters_and_values> )

ApproachLights( <list_of_parameters_and_values> )

-> FS98SDK

-

This command is very similar to RunwayData() but

is more elementary. When working with bitmaps you

may already have a photo of a runway so you probably

do not want to create a second one. This command

enables you to iluminate your bitmap runway.

Usually you will need one of this commands for each

runway end with the same heading. You must set the

'ThrLights' parameter to enable the other lights.

To make this command compatible with the RunwayData

command you should use a special scaling factor

(0.3048). The altitude setting must be done in the

RefPoint( 2 ... ).

Note: SCASM 1.6 has a bug in some of this parameters.

-

HEADing         heading in degrees, fractional

numbers allowed

LENgth          distance (position) of the threshold

lights measured from the center point

(usually half the runway length). Use

negative values for the near end.

Depends on the scaling factor

WIDth           runway width, depends on the scaling

factor

ThrLights       see -> ThrLights#

Strobes         number of strobes

VasiBars        see -> VasiBars#

VasiSlope       see -> VasiSlope#

VasiSide        see -> VasiSide#

negative values for near-left or far

right, positive values for near-right

or far left

VasiDist        see -> VasiDist#

negative values for near end,

positive for far end.

VasiSpace       see -> VasiSpace#

AprL            see -> AprL#

Building( delta_x  delta_z  delta_y

height  x  y  type  wallflags )

-

This command draws one of the FS5 internal standard

buildings. Normaly you should use an own RefPoint()

for each building.

You can rotate a building with RotatedCall().

-

delta_? delta coordinates from the active Reference

point, usually 0 (int). Caution, if you are

using large Delta_? values you may notice

some strange visibility effects with other

buildings nearby! For beginners I strongly

recommend to use zerovalues. Also each

building should have its own Reference point

and its own PerspectiveCall() instruction as

in the example.

height  the height of the building. It seems this is

measured in number of floors where each floor

is 4 refpoint units (int).

x, y    X/Y (east-west/north-south) dimensions (int)

of the building.

type    hex value, the lower 3 bits are coresponding

to the SIDEx.R8 bitmaps files. To get the

desired type add one value from each group.

00    beige with vertical stripes

01    white with vertical stripes

02    dark gray with vertical stripes

03    gray with vertical stripes

04    white with dark vertical stripes

05    white with horizontal stripes

06    beige with horizontal stripes

07    light gray with vertical stripes

The bits 3 and 4 defines the shape of the

building.

00    normal building.

08    inclined roof, roof pointing to the

north.

10    inclined walls, like a pyramid.

18    octogonal building, -> additional

wallbits needed.

8000    Instead the predefined SIDE#.R8

bitmaps a user defined bitmap is

used. ->bitmap()

Use the Bitmap() command just before

the Building().

Note 1: the bitmap must have a

special design.

Note 2: It seems that FS does not

allow to combine this option with

other's.

other bits unknown

wallflags       hex value. There is a bit for every

wall and the roof. Normaly set to

FFFF, 1FF or 1F.

0001    roof

0002    west

0004    east

0008    south

0010    north

0020    south-east

(needed for octogonal

Buildgs.)

0040    south-west ( -"- )

0080    north-east ( -"- )

0100    north-west ( -"- )

; example for the use of a rotated building with

; shadow

;

Area( 5 54:55:25.6584  8:20:19.7976 8 )

PerspectiveCall( :SubRoutine1 )

ShadowCall( :SubRoutine1a )

Jump( : )

:SubRoutine1

Perspective

:SubRoutine1a

RefPoint( rel :no_building  1

54:55:25.6584  8:20:19.7976 )

;  Note, since this refpoint lays in a subroutine

;  the label must NOT point to the end of the

;  block but to a Return command!

;

RotatedCall( :SubRoutine2 0 0 -3.5 )

:no_building

Return

;

:SubRoutine2

Building( 0 0 0  5 35 50  0 1F )

Return

EndA

City

-                                       FS5 .. FS98

This command generates a standard city, the same

buildings as on some FS5 Synth tiles.

;

; Example:

Area( 5 53:42:30  07:49:20  80 )

PerspectiveCall( :A )

Jump( : )

:A

Perspective

RefPoint( 7 :B 16  53:42:30 7:49:20  v1= 10000 )

City

:B

Return

EndA

IfPlaneInBox( :Label  xmin xmax  zmin zmax  ymin ymax )

-> FS98SDK

Monitor3D( :Label  xmin xmax  zmin zmax  ymin ymax )

-

The 3D area is monitored. If the aircraft is NOT in

the specified area a jump to :Label is performed.

The values are always in meters.

Note, this command cannot be roteted (RotatedCall) or

transformed (TransformCall) in FS5

see -> MonitorTr()

AreaSense(   :Label  ptx0 pty0  ...  ptxn ptyn )

-> FS98SDK

SenseBorder( :Label  ptx0 pty0  ...  ptxn ptyn )

-

The following instruction block is executed if we are

within the polygon defined by the list of point

coordinates. Oterwise execution is continued at

-> Label.

Concave polygones are not allowed and the points

should be entered in a clockwise order !

IfVis( :Label ptnum0 ... ptnumX )                -> FS98SDK

MonitorPt( :Label ptnum0 ... ptnumX )

-

maybe renamed in future versions! Describtion

incorrect.

-

Only found with 4 and 8 points!

-

This is a 3D monitoring instruction using point

numbers for defining the 3D area.

The following istruction block is executed if we are

within the polygon defined by the list of point

coordinates. Oterwise execution is continued at

-> Label.

Points have to be defined with the Points() command

before.

MonitorTr( :label

dx    dz    dy

wx    wz    wy

xdeg  ydeg  zdeg

)

-                      FS 5.1 & FS95 & FS98 & FS2K

This is the transformed 3D monitoring instruction.

The monitored 3D area is shifted (from the RefPoint

position) and rotated. A jump to :Label is performed

if the aircraft is outside.

-

dx      x offset from the actual reference point

(int)

dz      z offset from the actual reference point

(int)

dy      y offset from the actual reference point

(int)

wx      total x width of the area (int)

wz      total z width/height of the area (int)

wy      total y width of the area (int)

xdeg    x rotating angle

ydeg    y rotating angle

zdeg    z rotating angle

SurfaceType( type  width_x  width_y  alt )

-                                FS5 .. FS98,FS2K

This instruction lets you define the surface

properties of a given rectangular area. Note, you

cannot 'harden' an elevated surface with this

command! Use aditional section 16 commands for this.

-

type            0: smooth surface, runnway, taxiway

1: rough surface, grass

2: water surface, causes

splash / crash

width_x

width_y         N-E / N-S deviation, depends on the

scale factors in the RefPoint()

command. The width is measured from

the left (or upper) side to the

right (or lower) side. (integer)

alt             Altitude of the surface measured in

meters (integer).

In FS2K I found this parameter

always 0.

Inst_7D -> obsolete, use POverride

This instruction is used very often just before

TexPoly() and ShadedTexPoly(). It seems this

instruction prevents the bend up

effect of the bitmaps when getting close to it.

POverride                                        -> FS98SDK

-

Perspective Override

This instruction causes FS to use a special processor

for the next 3 or 4 vertex textured polygon. This

reduces the texture distortion.

For more information please see the FS98SDK.

TimerSw( :Label )    -> obsolete, don't longer use, vers 1.7

SetWeather( :Label )                             -> FS98SDK

-                                  FS5 .. FS98,FS2K

This instruction works like a timer controlled

switch. The following code is executed once about

every second to set new weather conditions. You can

test this with 3D objects.

The programm code that defines the lift must follow

imediately this instruction and must end with Return.

:Label points to an instruction where to continue,

after the lift is created or if the switch is not

active. This is usually the end of an Area() block.

-

example:

Area( 5 ... )

SetWeather( :end )

; TimerSw( :end ) ; old

Perspective

RefPoint( rel :end_lift ... )

IfPlaneInBox( :end_lift ... )  ; to limit the

; border of the

; lift area

...

Weather( 14C 200 6000 0 )      ; Lift in older

; versions

:end_lift

Return    ; very important !

;

:end

EndA

Lift(  flags  dir  factor  extra )       obsolete, vers. 1.7

Weather( flags dir factor  extra )               -> FS98SDK

-                                  FS5 .. FS98,FS2K

This is the instruction to change the weather

conditions. In the moment only thermal lift and lift

by deflected air are known. Thermal lift seems to

work always if the sun is shining<g>. Lift produced

by deflected air only works if surface wind is

enabled in the weather menu and the wind direction

is similar to the "dir" parameters. The strength

of the lift depends on the wind speed, wind direction

and the factor (of lift ?). Depending on the wind

speed (10..40 kts) the lift factor is typicaly from

6000 to 1000.

-

flags   hex value, typical values are 14C, 34C, 44C

first digit:

1..     enables lift created by deflected air

2..     enables turbulent air

4..     enables thermal lift. Thermal lift

depends on the time of day. It seems

the other parameters have no effect

on thermal lift.

.4C     always found this value. Setting it

to 00 or FF seems to do nothing.

dir     direction (heading in degrees) from where the

wind should come to produce a maximum of

lift. Only valid if "flags" are "1..".

factor  this factor is used to control the amount of

lift. Typical values are 2000 to 6000. The

format of this parameter may be changed in

future versions.

extra   unknown, set to 0.

LightBeamCall( :Label varptr vx vz vy )       -> SCASM 2.01

LandingLights( :Label varptr vx vz vy )         -> FS98SDK

-

This instruction is used to generate the aircrafts

landing lights.

-

varptr  points to a set of local variables which

holds the position data of this object.

vx

vz

vy      the vector components of the light beam

Nop

-

No Operation. This command does nothing. It only

occupies 2 bytes.

Usefull for patch programms or to make space for

later patches. Same as Dwx( 0x02 ).

PolyM( a                         pnum1 ... pnumn )

PolyM( m       vx vz vy   len    pnum1 ... pnumn )

PolyM( vattr  [vx vz vy   len]   pnum1 ... pnumn )

-                              SCASM 2.08,FS98,FS2K

This command draws a polygon similar to the Poly(..)

type. A SurfaceColor() should be defined first. This

polygon can be filled with a texture if the Bitmap()

command is used. The advantage of this command is,

that multible polygons can be drawn with this single

command and every polygon can have one "concave

point".

There is no need to use the ConcavePoly command, but

you have to start with the "concave point". This

point must be marked by a "-" sign, even if the

point index is zero!

Note: Due to the binary format in the BGL code it

is not possible to use point numbers larger than 127.

Example:

...

Points( 0

-327  0  861 ; 0

-246  0  792 ; 1

-101  0  619 ; 2

43  0  135 ; 3

309  0 -515 ; 4

328  0 -740 ; 5

269  0 -860 ; 6

272  0 -540 ; 7

-148  0  615 ; 8

-260  0  751 ; 9

)

SurfaceColor( 4 F0 )

IfVarAnd( :B 340 FFFF )  ; textured ground ?

Bitmap( desert.r8  8  128 0 128 )

:B

PolyM( a

-8 9 0 1 2 3

-7 8 3 4 5 6

)

...

The following 2 commands have experimental status (FS98).

Seed( s1 s2 obj xsize ysize  alt  s  w  r )

-

s1,s2,obj   see SCASEED.DOC

xsize       dimension in east-west direction

(meter, integer)

ysize       dimension in nort-south direction (")

alt         altitude (meter)

s           south edge (->SDK), whatever that means

w           west edge (-SDK)

r           random number (->SDK)

-

I implemented this instruction with the information

in the FS98SDK but I could not get it to work

correctly.

SeedAddObj( :Label )

-

This instruction adds a Seed() to the radsort queue.

So it seem to have the same effect to a Seed object

as a PerspectiveCall() to other objects.

If you want to experiment with these instructions you

can start with the following code:

;

;       Seed.scm

;

;       calling sequence:

;       Macro( seed.scm  Lat  Lon  alt )

;

;

Area( 5 %1  %2  5 )

SeedAddObj( :seed )

Jump( : )

;

:seed

Perspective

RefPoint( abs :endobj 1 %1 %2 v1= 32000 v2= 2000 )

Seed( 8 0 1111  500 500  %3  0 0 12345 )

:endobj

Return

EndA

Some commands to produce any hex-value in the visual scenery area. These are included to give experts a tool for trying new scenery commands. Note, the internal instruction counter variable $IC always contains the relative address of the first value in a multi D..() instruction.

Dlx( hhhhhhhh )         DefineLongHex. A 32 bit hex value is

inserted.

Dld( dddddddd )         DefineLongDecimal. A decimal value is

converted in 32 bit hex format and

inserted.

Dwx( hhhh .. hhhh )     DefineWordHex. All hex values in the

brackets are copied into the file.

Dwd( ddddd .. ddddd )   DefineWordDecimal. All decimal values

in the brackets are copied into the

file.

Dbx( hh .. hh )         DefineByteHex.

Dbd( dd .. dd )         DefineByteDecimal

Dba( AbCdE )            DefineByteASCII

Section 16 commands:

--------------------

Besides other records this section contains data structures similar to section 9. With the following instructions you can   define elevated surfaces above the ground level defined with Synth 1 ... Synth 6.

Area16( TopLat BottomLat RightLong LeftLong )

Area16( a Lat Lon ns-dist ew-dist )                vers 2.07

Area16( g ns-dist ew-dist )                        vers 2.07

-

Defines an rectangular area around your elevated

surface. Make sure that all used points are within

this rectangular area.

Atention: Due to the binary format of this command

the total length of one Area16()...End16 is limited

to 255 Bytes. Also the precision is limited, so it

does not make much sense to enter seconds in the

Lat/Long format.

-

There are 2 aditional formats for more flexible macro

processing.

a       automatic, the bounadaries are calculated by

using the given Lat/Lon values and adding the

ns/ew distances to this point.

example for a macro:

Area16( a %1 %2 5.5 5.5 )

If %1 and %2 are the Lat/Lon parameters in

this macro this defines an rectangle area of

totally 11 * 11 kilometers around this

location.

g       the same automatic as above, but no Lat/Lon

values are expected. These are taken from the

stored "general" reference point (-> GRP() ).

Area16( g 5.5 5.5 )

ns-dist distance in kilometers

The north and south boundaries are calculated

by adding this distance to both directions of

the reference point.

ew-dist distance in kilometers

The east and west boundaries are calculated

by adding this distance to both directions

of the reference point.

End16

-

defines the end of this area16 instruction block.

SetElevation( elev )

-

Defines an elevated hard surface i.e. for a runway on

a mountain. Elevated surfaces produced by this

command are always smooth (good for runways).

This instruction was found only in section 16 !

-

elev    elevation (MSL) in meters (integer).

SenseBorder( :Label  ptx0 pty0  ...  ptxn ptyn )

-

the same as in section 9

RefPoint( abs ... E= 0 )

-

the same as in section 9. Do not use other RefPoint

types. Sometimes it is usefull to set parameter V1

on large elevated surfaces.

RotatedCall( ... )

-

the same as in section 9,

Found in some original files but it seems this does

not work in section 16 in early FS versions.

Return

-

the same as in section 9

Jump( :Label )

-

the same as in section 9

IfVarRange[n]( :Label  var minval maxval )

-

the same as in section 9

IfVarAnd( :Label Var Mask )

-

the same as in section 9

CallAsm( :label hval )

-

the same as in section 9

dbx, dwx, dlx, dbd, dwd, dld

-

the same as in section 9

other commands are simply ignored, no error message !

Example for an elevated surface:

;

; This defines the hard surface of the platform in

; the joke.stn in my early GERISNS scenery.

;

Area16( 54:43  54:41:30   7:12  7:07 )

Refpoint( abs :  1  54:42:06  07:10:00  )

SenseBorder( :

-40      40      ; top left

40      40      ; top right

40     -40      ; bottom right

-40     -40      ; bottom left

)

; remember to enter the points in clockwise order

;

SetElevation( 30 )

End16

------------------------------------------------------------

Pseudo commands:

----------------

Pseudo commands are commands that do not produce any code. They are used to send aditional instructions or information to the assembler itself.

Include( filname.ext )

-

Includes this text file. The file is simply copied

into your 'main' source file. In SCASM 2.10 and later

an include file can contain other Include()'s or

Macro()'s.

Any correct path/filename is accepted (up to 260

characters).

This makes it possible to make a 'main' file like

Header( ... )

LatRange( ... )

Include( ... )

...

Include( ... )

with all the other commands in the Include() files.

Note, Include()'s are handled now as Macro()'s

without parameters. You cannot use them to increate

the nesting level.

Macro( filename.scm p1 ... pn )

-

Similar to include files the text of an macro file

is simply copied into your source file. But the

advantage of an macro is that parameters can be

passed to it. These parameters are inserted into the

text during compile time. This makes it very easy to

'recycle' scenery objects. If you want to design

a house, it is usually a good idea to do this as a

macro file and use the strings %1 and %2 for the

latitude and longitude values of its position.

Now you can put this house into your scenery simply

by puting its actual Lat/Lon position as parameters

one and two into the Macro() instruction. SCASM

allows up to 30 parameters and each can have 15

characters.

Macros are allowed in visual scenery (section 9),

dynamic scenery (section 15) and section 16.

In SCASM 2.10 and later macros are allowed to contain

other macros. This nesting is limited to 8 levels,

and Include()'s are handled now as macros without

parameters.

If a macro (file) name or a macro parameter contain

spaces you have to use " to mark the begin and end of

it (SCASM 2.15 and later).

example:

macro call:     macro( example.scm N54:10 E10:05 )

macro file:     Area( 5 %1 %2 20 )

-> %1 is replaced by N54:10

-> %2 is replaced by E10:05

Note: In most cases macros will contain a whole

Area() block. But if you are using the macro feature

to generate some code within an area (for example

'hand made' aproach lights in an runway area), and

if you are using the same macro more than once in

the same area, and if the macro contains label

definitions you will get an "duplicated label error".

To avoid this you can include the special '@'

character in the label name. This character causes

SCASM to insert the internal macro counter in the

Label name. Since the internal macro counter is a

3 digit code the name is then limited to 12

characters.

example:

:Label@ is expanded to :Label095 during the

expansion of the

95th macro.

Mif( <expression> )

Melse

Mifend

-

These pseudo commands are used to enable and control

conditional compilations. In earlier SCASM versions

they are only used in macros for visual scenery.

That's why they have the "M" in the name. Now they

can be used anywhere in the source text and they

even can be nested (up to 8 levels).

<expression> can be a mathematical formula or a

single number.

-

mif( %1 )

mif( [%2 == 3] )

The macro parameter 1 is tested.

If it's value is NOT 0

(that is TRUE) the following commands

are compiled. Only a zerotest is

done.

In example 2 the expression

"[%2 == 3]" is evaluated and then

tested.

see -> annex 1 for expressions and

operators.

melse           Optional. If the 'mif( %# )'

condition is FALSE

this section is compiled by the

assembler.

mifend          Marks the end of the conditional

compilation.

If an error is detected and the compiling condition

is FALSE, then the error messages will have the line

number of the Mifend command.

CopyRight( any text with up to 80 characters )

-

This text is inserted into the binary output file so

you can identify your work with an hex file viewer.

It has no effect to FS. This command should be used

only once per source/BGL file. If this command is

found more than once, only the last one is used

(no error message). For this reason it is not a good

idea to use it in a macro file.

No underscores '_' are needed for SPACE characters in

the text.

Uvar( $name value )                           1.7 and later

-

This is an internal command to set an SCASM internal

user variable. This command has no effect to the BGL

code. User variables can be used by experts for more

flexible parameter calculations.

Note: variables which are defined within an Area()

are only valid in this Area() and are deleted when

the EndA statement is compiled. Other variables are

always valid.

SCASM can handle up to 100 user variables.

Map( stat )

-

This is another internal command which can be used to

suppress the map file generation for some parts of

the source code file.

See also the "-m" command line option.

-

stat	1 = on,

0 = off

-

For more information please see the documentation

which comes with the BGLTST.EXE program which uses

this information to display the source code line(s)

related to an detected error.

Error( any error message )

-

This command is an pseudo command, which means that

it does not generate any BGL code. It produces an

error message and stops SCASM. Some experts asked me

to include this instruction.

Can also be used as a reminder for changes you want

to do later. This command can also be used to avoid

the compilation of a macro .API file with an obsolete

compiler version. For example, if internal functions

(version 2.02 and later) are used for the parameter

calculations, you can use this instruction to notify

the user that he needs an compiler

update:

...

mif( [$Version < 202] )

Error( You need at least SCASM 2.02 to

compile this code )

mifend

...

Points( 0

-15  0  int[ %6 / 2 ]

...

)

...

In this example the macro receives a parameter from

the caller. If the %6 macro parameter has the value

15 the result would be 7.5. Since FS does not accept

fractional numbers in point coordinates, an error

message is normally generated. The int[] or the

round[] functions can now be used to convert the

number format but these are not available in earlier

versions.

VTrace( mask "any text" <expression> )

-

This is an SCASM internal test command for debugging.

It does not produce any code. You can use this

command if you want to know the result of of some

SCASM calculations.

The output is done to the standard error device,

which is the screen or the "SCAERROR.LOG" file.

-

mask         This is a bitmask in HEX format to

indicate the section when this command

shal be executed. If you are unsure

about this value set it to -1

(or 200 for section 9, visual scenery).

"any text"   Any text you want to see on the output

device. (max 120 chraracters)

<expression>	Any valid expression in SCASM syntax.

-> see annex for more information

Set( flag value )

-

table of flag names:

interr    Same as -i switch, 1 = ignore integer

errors. Default = 0

fmtcast   Same as -f switch, 0 = format casting

disabled. Default = 1

ppperr    Number of points per polygon to trigger the

error message. Default = 100.

areamx    Maximum allowed size of an Area() block in

KB. Default = 16, maximum 64.

area15mx  Maximum allowed size of an Area15() block

in KB. Default = 16, maximum 64.

RAW       1 | 0 enables or disables the -RAW switch

-> -raw

LOGFILE   1 enables error output to SCAERROR.LOG

-> -l

MAPFILE   1 enables the mapfile (.MPF) output

-> -m

OBJID     This flag controls the object ID generator

when object libraries are compiled (2.36+).

1 (default) ID0 value is incremented

0 ID3 value is incremented. Only needed

for backward compatibility.

-

Some users wanted to have access to SCASM internal

flags and error threshold values. This can be done

with this command.

A change of some of these values may probably result

in display problems in some FS versions.

This command should be placed at the beginning of the

source code file (before any other command causes

dynamic memory allocation) and is not recognised if

placed in macro files. This is because macros are not

completely scanned during the first source scan.

------------------------------------------------------------