Some new commands

-----------------

ILS2(   Frq     Rng     Flags   Magvar

ID      Name

Lat     Lon     Elev    Hdg              ; localizer

Lat     Lon     Elev    Slope            ; GP

X                                        ; unknown

)

-                                   SCASM 2.34/FS2K

Defines an ILS (instrument landing system)

-

Frq     frequency in mc -> 113.45

freq. range VOR 108.0 ... 117.975 mc

freq. range ILS 108.0 ... 111.975 mc

Rng     range in nm (integer)

Note: you can enter values up to 255.

It seems FS5 can handle this, but the DME

display will always show "99" if this

value is exceeded.

Flags   any 2 digit hex value is accepted

80 localizer on ON bit

40 glideslope transmitter ON bit

20 localiser back course not available

10 DME at glideslope transmitter position bit

08 DME OFF bit (-> 09 is DME without VOR)

04 voice available (ID ? ATIS ?)

02 TACAN

01 DME availabe bit

example values:

00 or 02 -> VOR

01 or 03 -> VOR-DME, VORTAC

Magvar  magnetic variation (eg.-3.7). Only for VOR.

Normally 0 for ILS. If 0 the VOR/ILS is

aligned with true north.

To get a nav receiver readout which

corresponds to your compass you should enter

the magnetic variation. This gives the VOR

the same error than your compass.

ID      ID string, up to 5 characters

Name    Name string, up to 24 characters.

Use '_' in place of ' '.

Lat     latitude

Lon     longitude

Elev    elevation in meters (MSL, integer)

Hdg     heading (true) in degrees (ILS only,

always 0 for VOR) -> 144.1

Slope   slope angle of GP transmitter -> 3.0

Note: There is a known problem with the slope

in the southern hemisphere in FS5.0 and

FS5.0a.

X       unknown

This parameter is new in FS2K.

Currently this parameter is implemented as

"short integer".

Found values range from 0x1FF to 0x68C

(511 - 1676). Maybe this is some sort of

angle. If these values are asumed to be

coded as 16 bit pseudodegrees the range is

from 2.8 to 9.2 degrees.

Remark: Not all ILS systems have an DME equipment.

In the USA DME's usually show the localiser

distance.

In Europe DME's usually show the touch down

zone (TDZ) distance (position of the GP

antenna).

DMEPos( Frq  flg  Lat Lon  elevation )

-                                             FS98

This command was first found in FS98. It was not

mentioned in the SCASM documents because I am still

unsure about some of its parameters.

Therefore it may become necessary to change the

format later.

This command must immediately follow the associated

ILS()/ILS2() command and must have the same

frequency as the ILS. Be sure that the asociated

ILS has the "DME available" flag set.

-

Frq         The frequency of the ILS system

The frequency is not coded in the command

but is needed for sorting purposes in

all navaid objects.

flg         unknown

always found 1

Lat         Latitude

Lon         Longitude

elevation   elevation (integer, MSL) in meter

Temporary new commands for section 16:

Area16N

-                                             FS2K

New type of Area16 command

This one has no parameters and should only

contain one BGL instruction.

The end is marked with the already known End16

command.

Elevation ( elevation <list of LAT/LON values> )

-                                             FS2K

This command defines an elevated surface.

It seems that FS2K uses at least one

Area16N - Elevation - End16

sequence for every airport. In adition to this

the SurfaceType() command is used in section 9

to make the surface smooth.

The elevation is wanted in meters.

In FS I often found 3 to 9 Lat/Lon values, mostly

in anti clockwise order and often even for concave

areas.

-

Example:

Area16N

Elevation ( 488.2895966

33:22  -118:28

33:22  -118:22

33:27  -118:22

33:27  -118:28

)

End16

Notes:

* FS2K uses Z-Buffering on polygon level wheras previous versions did this on object level. This is why you normally do not longer need VectorJumps() in objects to avoid bleed through effects. But it seems that single points and lines are not sorted in the Z-Buffer. If you have bleed through problems with these elements, try to solve them by using VectorJump() commands as we did it in the past.

* The "endless loop" bug Pascal reported, when unknown identifiers/macro-parameters are referenced is hopefully fixed.

*Experimental version of the new buildings added in 2.35

*Flags added to the type parameter in LoadBitmap()

*Library object ID handling modified.

* 2.38 AFD BGL's are not longer limited to a total size of 160 kB

FS2K scenery design problems

----------------------------

1) ground polygons drawn with:

StartSurface

MoveToPt()

DrawToPt(...)

EndSurface

are not displayed. Replace them by:

Poly( au ... )

2) It seems that MS used only RefPoint( rel ... ) in

their files.

*) During the installation of new scenery it sometimes happens that the file SCENERY.CFG was set to "write protected". The result is that this file cannot be updated and FS2K will not show the scenery. Use Windows Explorer/Properties to uncheck the "read only" file attribute.